# SAT6

## ADCS structure

General: ADS -> ACS -> [Mag.Torq,  Reac.Wheel ] 
(only work on ACS has been made)

ACS:   (only target tracking, and by proxy Nadir, has been developed)

LQR & PI for linear ACS

SMC for nonlinear

## Sensors/actuator

Camera

Reaction wheels

-dynamic model
-wheel configuration

## Disturbance rejection

Solar radiation
Air drag
Magnetic disturbance
Gravity gradient: neglected when other disturbances are maximized for disturbance rejection

## Conclusion

Quaternion SMC:
- linear and nonlinear success for point tracking.
- Coriolis moment was not compensated, was assumed small enough to be neglected.
MEKF:
- succeeded all requirements
Attitude control testbed:
- partial success. Can utilize HIL for debugging and verification, has issues for mass centering.
SMC implementation:
- for single axis, the SMC was succesfull.
Future:
- Fix testbed for multi-axis control

# SAT5

ISS orbit (<1 year lifetime). Payload: AIS system

## ADCS structure

## Sensors/Actuators

Sun sensors

Gyroscopes

Magnetometer

Magnetorquer

## Disturbance rejection

-Magnetometer influencing sensors etc. (controlled by setting a max on duty-cycling = 88%)
-Solar radiation pressure and winds

## Conclusion

-control is successful for single axis with constant magnetic field
-control was unsuccessful for multi-axis space environment. 3 assumptions for cause of failure
 	-Assumed to be because of single-axial actuation influencing other axes. 
 	-Inertia matrix of system is not diagonal, reinforcing first assumption
	-varying magnetic fields, as would be with earth’s magnetic field.

The satellite was sent in october 2015. It was difficult to communicate with it, because of a change in its orbit. After relaying from a volunteer radio enthusiast in germany, the satellite returned to normal operation.

Fun Fact: The satellite was built as a request of ESA for the first danish astronaut.

# POST-Launch

Launched: 19/8-15
Currently: Re-entered atmosphere 15/3-16

- Issues with communication, solved afterwards

- No issues mentioned pertaining to ADCS.

# SAT4

Payload: AIS system.

Launch: April 2016

Windings: 275					Dimensions: 0.075m x 0.075m
Output: 0.5-1.1 µNm (at 700km altitude)

Windings: 275					Dimensions: 0.075m x 0.075m
Output: 0.5-1.1 µNm (at 700km altitude)

ACDS structure
Segmented into two ACDS-(1 for detumbling, 2 for ACS)

MODES: detumbling, point 
Processor for ACDS2: ARM7 (32bit, 600MHz)
Uses Blackfin processors for Automatic Identification System circuit (need to find out what AIS does).
Sensors/Actuators
Magnetorquer

Magnetometer				   	(other reports state a MAG3110FS)
operating freq: 80Hz
sensitivity: 0.1 µT

Gyroscope LPY403A
- resolution: up to 120 deg/s
- sample rate: 140 Hz

SAT4: pointing control uses 2 gyros??? Also uses an ADC for data conversion.

Sun sensor
Inhouse built, stores energy in solar cells

# Disturbance rejection

# Conclusions

## Attitude control system (Brian and Jens, bachelor)

- Magnetorquers bad for 3-axis control, but they’re cheap and easy to use. Effective in single axis control.

- Used a VICON system for ADS. ASK ANDERS LA-COUR ABOUT IT, HE IS SOURCED FOR IT.

- Requirements partial successful

## Att. Det. & point control sys.

- ADS worked, however not within +-2% boundary of error estimate.

- Pointing partially worked, nadir was not a success. Actuation speed was not obtained.

- Further improvements: use non-linear point control, use more complex ADS methods.

# POST-Launch

Launched 25/04-16
Currently: Operational

# SAT3

The objective of AAUSAT3 is to impletent an Automatic Identification System(AIS) for surveillance of Greenlandish waters. It also contains a GPS and a camera.

It has: ADCS1 that focuses on detumbling. ADCS 2 that contains everything

Launch: February 2013

# Modes of operation

# Sensors & Actuators

Magnetometers, GPS, gyros, sun sensors, temperature sensors.

# Error handling

Pretty good. It adds to types of FDI: simple for testing if sensors are operational and Model Based with more complex methods.

# Conclusion

Mission reached end of life. Properly working for more than 18 months. Still in orbit, last contact 2019.

Battery consumption exceeded the 1W restriction, needing up to 2W of power. Same with the payload, 1.1W. The solar panels showed constant degradation of generated power. They theorise it was due to atomic oxygen. AAUSAT4 appears to have solved this issue.

Tracking data

# SAT2

Mission payload: gamma ray burst detector

# Modes of operation

- Detumbling

- Idle: Minimum power consumption

- Active pointing mode

# Sensors & Actuators

“The ADCS HW sensor and actuator system consists of three momentum wheels, three magnetorquers, one three-axis magnetometer, six one-axes gyros, six photodiodes (also studied using the solar cells), and six temperature sensors.”

# Error handling

Most extensive definition and control out of all the other satellites. A must to read for next AAUSATs.

# Conclusion

Constant restarts of the satellite, around 10/day. The detumbler doesn’t do its job, satellite tumbles unless detumbled manually.

# SAT1

The payload of this satellite is a camera to be pointed to earth.

# Modes of operation

# Sensors & actuators

Sensors: Sun sensors & magnetometers

# Error handling:

# Conclusion:

Magnetorquers are too power consuming. The flywheel planned has been removed because of a lack of space. Sun sensors can provide an attitude error smaller than 4º with appropriate temperature control.

The batteries started failing quite quickly. We found no information of the ADCS performance.